EL22MGI V.3                                2-Port RJ45 +2G SFP L2 Managed Industrial Switch

Interface

- 2*10/100/1000BaseT(X), Automatic detection, Full/half duplex MDI/MDI-x adaptive

- 2*100/1000Base-FX Port SFP

Industrial conformance

- 9V to 52V DC, redundant power input

- -40℃ to 75℃ operating temperature

- IP40 aluminum housing

- Supports 6KV DC Ethernet ESD protection

- Supports 6KV DC EFT protection for power line

- Free Fall, Shock and Vibration Stability

- DIN-rail hardware design

EL22MGI(V.3) is a photoelectric convergence integrated Industrial Ethernet Gigabit Managed switch .This product is mainly developed for the application of point-to-point network, star network, tree network, chain network and ring network in the industrial field. With 4 10/100/1000Base-TX ports, 2 Gigabit SFP optical ports, all ports support full wire-speed forwarding, and its high-performance switching engine, and has a redundant self-healing ring (fault recovery time＜20ms) to realize single loop, tangent, and intersection loop protocols, support for broadcast storm suppression, port mirroring, port priority QoS settings, port aggregation functions. IP40 rated enclosure, fanless design with highly efficient wavy aluminum alloy shell heat dissipation surface, overcurrent, overvoltage and EMC protection for power inputs, excellent EMC protection for RJ45 ports, to make products work efficiently in harsh industrial environments. And it can provide a reliable and convenient solution for the users’ Ethernet equipment networking.

SPECIFICATIONS:

| Product Name | 2-Port RJ45 +2G SFP L2 Managed Industrial Switch |
| --- | --- |
| Model No. | EL22MGI V.3 |
| Ethernet interface |  |
| RJ45 Port | 2*10/100/1000BaseT(X),Automatic detection, Full/half duplex MDI/MDI-x adaptive |
| Optical port | 2*100/1000Base-FX SFP |
| Power Parameters |  |
| Power | DC9 to 52V redundant power support reverse connection protection |
| LED Indicator | Power indicator LED : Red PWR System indicator LED: Green SYS RJ45(Green) Link/Act SFP(Green) Ulink/Act |
| Standards |  |
| Network Protocols | IEEE802.3-10BaseT,IEEE802.3u-100BaseTX/100Base-FX,IEEE802.3x-Flow Control,IEEE802.3z-1000BaseLX,IEEE802.3ab-1000BaseTX,IEEE802.1ab Logical Link Discovery Protocol,IEEE802.1D-Spanning Tree Protocol,IEEE802.1w-Rapid Spanning Tree Protocol, APS ,IEEE802.1Q -VLAN Tagging, IEEE802.1p -Class of Service,IEEE802.1X-Port Based Network Access Control |
| Twisted Pair | 10BASE-T: Cat3,4,5 UTP(≤100 meter) 100BASE-TX: Cat5 or later UTP(≤100 meter) 1000BASE-TX: Cat6 or later UTP(≤100 meter) |
| Switch Properties |  |
| Processing Type: | Store and Forward |
| Packet forwarding rate | 8.928Mpps |
| MAC Table Size | 8K |
| Packet Buffer Size | 4.1Mbit |
| Bandwidth | 12G |
| Switching Delay | ＜5μs |
| Power Consume | ＜12W |
| Physical Characteristics |  |
| Protection level | IP40 |
| Enclosure | Wavy aluminum alloy |
| Installation | DIN rail |
| Weigh | 0.66kg |
| Dimensions | 140x104x42mm (L*W*H) |
| Environmental Limits |  |
| Working temperature | -40～75°C（-40～167°F） |
| Storage Temperature (package included) | -40 to 85°C (-40 to 185°F) |
| Ambient Relative Humidity | 5 to 95% (non-condensing) |
| Standards and Certifications |  |
| Safety | EN 62368-1(LVD) |
| EMI | FCC Part 15 Subpart B Class A EN 55032 Class A |
| EMS | IEC(EN)61000-4-2(ESD): ±8kV Contact, ±8kV Indirect,±15kV Air IEC(EN)61000-4-3(RS):10V/m(80～1000MHz) IEC(EN)61000-4-4(EFT): power line:±4kV; data line:±2kV IEC(EN)61000-4-5(Surge): power:±6kV CM/±4kV DM; data:±4kV IEC(EN)61000-4-6(RF conduction):3V(10kHz～150kHz),10V(150kHz～80MHz) IEC(EN)61000-4-16(Common mode conduction):30V cont. 300V,1s IEC(EN )61000-4-8 |
| Shock | IEC 60068-2-27 |
| Freefall | IEC 60068-2-32 |
| Vibration | IEC 60068-2-6 |
| MTBF |  |
| Time | 300 000hrs |
| Warranty |  |
| Warranty Period | 5years |

SOFTWARE:

| Port Features | IEEE 802.3x Flow control |
| --- | --- |
|  | Interface counters |
|  | Storm Control |
| Link Aggregation | Static Aggregation |
|  | Dynamic Aggregation, LACP |
|  | Supports up to 4 aggregation groups, each group up to 8 ports Load balancing |
| MAC Address Table | Static MAC Address MAC Binding |
| Mirroring | Port-based One-to-one Mirroring, Many-to-one Mirroring |
| Spanning Tree | IEEE802.1d STP |
|  | IEEE802.1w RSTP |
|  | IEEE802.1s MSTP |
| ERPS (G.8032) | Single Ring, Sub-Ring, Multi-Ring |
|  | Recovery Time < 50ms |
| LLDP | LLDP |
|  | LLDP-MED |
| Loop Protection | Supported |
| VLAN | 4094 VLAN |
|  | Port-based VLAN |
|  | GVRP |
| IGMP | Snooping IGMP V1 Snooping |
|  | IGMP V2 Snooping |
|  | IGMP V3 Snooping |
|  | Source Port Check |
|  | IGMP Fast Leave |
| Security |  |
| ACL | ACL (IP Standard ACL), IPv4/IPv6 |
|  | MAC extend ACL |
|  | IP extend ACL |
|  | L2, L3, L4 fields match |
| QoS | QoS Class, Remarking |
|  | SP, WRR queue scheduling |
|  | Ingress port-based rate-limit |
|  | Ingress priority-based rate-limit |
|  | Egress port-based rate-limit |
|  | Egress queue-based rate-limit |
|  | Policy-based QoS |
|  | Storm Control |
|  | 8 queues per port |
| 802.1X | Port-based |
| User management | Password protection |
| SNMP | SNMP V1/V2c/V3 |
| RMON | Statistics Group |
|  | History Group |
|  | Alarm Group |
|  | Events Group |
| Web management | HTTP V1.1, HTTPS |
| Firmware Upgrade | Reversion Upgrade |
| CLI | Support |
| Syslog/Debug | Support |
| Diagnosis | Ping (IPv4), ping (IPv6) |
|  | Trace route(IPv4) |
| Management |  |
| NTP | SNTP |
| Telnet | Telnet Server |
| TFTP | TFTP Client |
| SSH | SSH Server |

Order Information

| Specification | Model No. |
| --- | --- |
| L2 Managed Industrial Switch 2 Port 10/100/1000Base TX+2 Port 100/1000 SFP | EL22MGI (V.3) |