EL1021F series                                2-Port RJ45+1 optical Fast Industrial Media Converter

Interface

- 2*10/100BaseT(X),Automatic detection, Full/half duplex MDI/MDI-x adaptive

- 1*100Base-FX Port (SC/FC/ST/SFP optional)

Industrial conformance

- 9V to 52V DC, redundant power input

- -40℃ to 75℃ operating temperature

- IP40 aluminum housing

- Supports 6KV DC Ethernet ESD protection

- Supports 6KV DC EFT protection for power line

- Free Fall, Shock and Vibration Stability

- DIN-rail hardware design

EL1021F is an optical convergence integrated Industrial Ethernet Media Converter independently developed by our company. This product is mainly developed for the application of point-to-point networks, star networks, tree networks, and chain networks in the industrial field. It support 2-Port 10/100Base-TX and 1-Port 100Base-FX, optional SFP, SC, ST, and FC interface, and the longest optical communication distance can reach 120KM. 
In addition to its compact size, it has a high-performance switching engine, a wavy aluminum alloy shell with IP40 protection level, a fanless design and efficient heat dissipation surface, and an installed power input terminal over-current, over-voltage, the EL1021F Series features DC9-52 V redundant power inputs, DIN-rail mounting, high-level EMI/EMC capability, and an operating temperature range of -40°C to 75°C available. The EL1021F Series has also passed a 100% burn-in test to ensure it will function reliably in the field. Therefore, it can provide reliable and convenient solutions for users’ Ethernet equipment networking.

SPECIFICATIONS:

| Product Name | 2-Port RJ45+1-Port optical Fast Industrial Media Converter |
| --- | --- |
| Model No. | EL1021F |
| Ethernet interface |  |
| RJ45 Port | 2*10/100BaseT(X),Automatic detection, Full/half duplex MDI/MDI-x adaptive |
| Optical port | 1*100Base-FX Port (SC/FC/ST/SFP optional) |
| Power Parameters |  |
| Power | Connection: 1 removable 5-contact terminal block(s) Input voltage: DC9 to 52V redundant power support reverse connection protection |
| LED Indicator | Power: Green Link: Green |
| Standards |  |
| Network Protocols | IEEE802.3 10BASE-T;100BASE-T IEEE802.3i 10Base-T;IEEE802.3u;IEEE802.3x; |
| Twisted Pair | 10BASE-T: Cat3,4,5 UTP(≤100 meter) 100BASE-TX: Cat5 or later UTP(≤100 meter) |
| Switch Properties |  |
| Processing Type: | Store and Forward |
| MAC Table Size | 2K |
| Packet Buffer Size | 1Mbit |
| Bandwidth | 1.6G |
| Switching Delay | ＜10μs |
| Power Consume | ＜3W |
| Physical Characteristics |  |
| Protection level | IP40 |
| Enclosure | Wavy aluminum alloy |
| Installation | Push-in DIN rail |
| Weigh | 0.4kg |
| Dimensions | 115x86x30mm (length x width x height) |
| Environmental Limits |  |
| Working temperature | -40～75℃ （-40～167°F） |
| Storage Temperature (package included) | -40 to 85°C (-40 to 185°F) |
| Ambient Relative Humidity | 5 to 95% (non-condensing) |
| Standards and Certifications |  |
| Safety | EN 62368-1(LVD) |
| EMI | FCC Part 15 Subpart B Class A，EN 55032 Class A |
| EMS | IEC(EN)61000-4-2(ESD): ±8kV Contact, ±8kV Indirect,±15kV Air IEC(EN)61000-4-3(RS):10V/m(80～1000MHz) IEC(EN)61000-4-4(EFT): power line:±4kV; data line:±2kV IEC(EN)61000-4-5(Surge): power:±6kV CM/±4kV DM; data:±4kV IEC(EN)61000-4-6(RF conduction):3V(10kHz～150kHz),10V(150kHz～80MHz) IEC(EN)61000-4-16(Common mode conduction):30V cont. 300V,1s IEC(EN )61000-4-8 |
| Shock | IEC 60068-2-27 |
| Freefall | IEC 60068-2-32 |
| Vibration | IEC 60068-2-6 |
| MTBF |  |
| Time | 300 000hrs |
| Warranty |  |
| Warranty Period | 5years |

Installation method

Order Information

| Specification | Model No. |
| --- | --- |
| 2 Port 10/100Base TX+1 Port 100 Base FX SFP Industrial media convertor | EL1021F-SFP |
| 2 Port 10/100Base TX+1 Port 100 Base FX Port(multi-mode SC connector) 2KM distance | EL1021F-SC2 |
| 2 Port 10/100Base TX+1 Port 100 Base FX Port (single-mode SC connector) 20KM distance | EL1021F-SC20 EL1021F-SC20A / SC20B |
| 2 Port 10/100Base TX+1 Port 100 Base FX Port (multi-mode ST connector)2KM distance | EL1021F-ST2 |
| 2 Port 10/100Base TX+1 Port 100 Base FX Port (single-mode ST connector) 20KM distance | EL1021F-ST20 EL1021F-ST20A / ST20B |
| 2 Port 10/100Base TX+1 Port 100 Base FX Port (multi-mode FC connector)2KM distance | EL1021F-FC2 |
| 2 Port 10/100Base TX+1 Port 100 Base FX Port (single-mode FC connector) 20KM distance | EL1021F-FC20 EL1021F-FC20A / FC20B |