EL1612GSMI

16-Port SFP+8G RJ45+4G combo Managed Industrial Switch

Interface

- 16-Port full 1.25G SFP

- 8-Port 10/100/1000 RJ45

- 4G combo (4G RJ45 or 4G SFP) uplink

Industrial conformance

- Dual power input: DC36-72V, AC100-240V

- -40℃ to 65℃ operating temperature

- IP40 aluminum housing

- Supports 6KV DC Ethernet ESD protection

- Supports 6KV DC EFT protection for power line

- Free Fall, Shock and Vibration Stability

- 19” Rak-mountable hardware design

EL1612GSMI an hardened 16 Port optical L2 Managed Industrial Switch, provides non-blocking wire-speed performance in harsh industrial environment. It also provides 16 Port 10/100/1000BASE-FX SFP +8*10/100/1000 RJ45+4G combo uplink delivered in an IP40 rugged, strong case with redundant power system. It’s suitable for such applications as surveillance system deployments, control management and wireless service in climatically demanding environments with wide temperature range from -40℃ to 75℃.

The EL1612GSMI provides the flexibility to all kinds of 10/100/1000Mbps Ethernet Media on SFP port and performs highly stable fiber performance

The EL1612GSMI is packaged in a compact IP40 case that allows bracket mounting to cabinet. It provides dual power supply input DC36-72V and AC100-240V input

SPECIFICATIONS:

| Model No. | EL1612GSMI |
| --- | --- |
| 1000Base-FX Port | 16 Port SFP |
| 1000Base-TX | 8 Port RJ45 |
| 1000Base-TX/FX Port | 4G combo (either 4G RJ45 or 4G SFP) |
| Performance Specification | Bandwidth: 256Gbps Packet Buffer Memory: 16Mbit Packet Forwarding Rate: 131Mpps MAC Address Table: 16K |
| Installation | Rak mounted |
| Maximum Frame Size | 10240bytes packet size |
| Flow Control | Back pressure for half duplex, IEEE 802.3x pause frame for full duplex |
| Enclosure | IP40 Aluminum housing |
| LED Indicator | Power: Red Fiber: Link (Green) Ethernet: Yellow |
| Power Input | DC36-72V, AC100-240V |
| Power Consumption | < 30W |
| Surge protection | ± 4KV |
| Network Protocols | IEEE802.3 10BASE-T; IEEE802.3i 10Base-T; IEEE802.3u;100Base-TX/FX; IEEE802.3ab 1000Base-T; IEEE802.3z 1000Base-X; IEEE802.3x; |
| Network cables | 10BASE-T: Cat3,4,5 UTP(≤100 meter) 100BASE-TX: Cat5 or later UTP(≤100meter) 1000BASE-TX: Cat6 or later UTP(≤100 meter) |
| Industry Standard | FCC CFR47 Part 15,EN55032 Class A IEC61000-4-2 (ESD): ±8kV (contact), ±12kV (air) IEC61000-4-3 (RS): 10V/m (80~1000MHz) IEC61000-4-4 (EFT): Power Port: ±4kV; Data Port: ±2kV IEC61000-4-5 (Surge): Power Port: ±2kV/DM, ±4kV/CM; Data Port: ±2kV IEC61000-4-6 (CS): 3V (10kHz-150kHz); 10V (150kHz-80MHz) IEC61000-4-16 (Common mode conduction): 30V (cont.), 300V (1s) |
| MTBF | 300,000hours |
| Dimensions (W x D x H) | 442×325×44mm |
| Weight | Product Weight: 4KG Packing Weight: 4.5KG |
| Working Environment | Working temperature: -40～75℃ ; Storage temperature: -40～75℃ Relative Humidity: 5%~95 %( no condensation) |

SOFTWARE:

| Port Features | IEEE 802.3x Flow control |
| --- | --- |
|  | Interface counters |
|  | Storm Control |
| Link Aggregation | Static Aggregation |
|  | Dynamic Aggregation, LACP |
|  | Supports up to 4 aggregation groups, each group up to 8 ports Load balancing |
| MAC Address Table | Static MAC Address MAC Binding |
| Mirroring | Port-based One-to-one Mirroring, Many-to-one Mirroring |
| Spanning Tree | IEEE802.1d STP |
|  | IEEE802.1w RSTP |
|  | IEEE802.1s MSTP |
| ERPS (G.8032) | Single Ring, Sub-Ring, Multi-Ring |
|  | Recovery Time < 50ms |
| LLDP | LLDP |
|  | LLDP-MED |
| Loop Protection | Supported |
| VLAN | 4094 VLAN |
|  | Port-based VLAN |
|  | GVRP |
| Protocol VLAN | MAC-based VLAN |
|  | IP Subnet-based VLAN |
|  | Voice VLAN |
| Port Isolate | Support |
| IGMP | Snooping IGMP V1 Snooping |
|  | IGMP V2 Snooping |
|  | IGMP V3 Snooping |
|  | Source Port Check |
|  | IGMP Fast Leave |
|  | IGMP Filter |
| MLD Snooping | MLD V1 Snooping |
|  | MLD V2 Snooping |
|  | Source Port Check |
|  | MLD Fast Leave |
|  | MLD Filter |
| Security |  |
| ACL | ACL (IP Standard ACL), IPv4/IPv6 |
|  | MAC extend ACL |
|  | IP extend ACL |
|  | L2, L3, L4 fields match |
|  | log, redirection, mirroring, speed limit |
| QoS | QoS Class, Remarking |
|  | SP, WRR queue scheduling |
|  | Ingress port-based rate-limit |
|  | Ingress priority-based rate-limit |
|  | Egress port-based rate-limit |
|  | Egress queue-based rate-limit |
|  | Policy-based QoS |
|  | Storm Control |
|  | 8 queues per port |
| 802.1X | Port-based |
| User management | Password protection |
| SNMP | SNMP V1/V2c/V3 |
| RMON | Statistics Group |
|  | History Group |
|  | Alarm Group |
|  | Events Group |
| Web management | HTTP V1.1, HTTPS |
| Firmware Upgrade | Reversion Upgrade |
| IPv4/IPv6 | Support |
| CLI | Support |
| Syslog/Debug | Support |
| Diagnosis | Ping (IPv4), ping (IPv6) |
|  | Trace route(IPv4) |
| Management |  |
| NTP | NTP Client |
| DHCP | DHCPv4 Client, DHCPv6 Client |
| DHCP Snooping | DHCP snooping table |
|  | Trust Port |
| Telnet | Telnet Server |
| TFTP | TFTP Client |
| SSH | SSH Server |

Ordering Information

| EL1612GSMI | 16 Port Full gigabit optical+8G RJ45+4G combo Managed Industrial Switch |
| --- | --- |