EL10822FG-SFP                                8-Port 10/100 RJ45+2G RJ45+2G SFP Industrial Switch

Interface

- 8*10/100BaseT(X),Automatic detection, Full/half duplex MDI/MDI-x adaptive

- 2*1000Base-T RJ45

- 2*1000Base-F SFP

Industrial conformance

- 12V to 52V DC, redundant power input

- -40℃ to 75℃ operating temperature

- IP40 aluminum housing

- Supports 6KV DC Ethernet ESD protection

- Supports 6KV DC EFT protection for power line

- Free Fall, Shock and Vibration Stability

- DIN-rail hardware design

EL10822FG-SFP is a photoelectric convergence integrated industrial Ethernet seamless redundant ring switch independently developed by our company. The product is mainly developed for the application of point-to-point network, star network, tree network, chain network and ring network in the industrial field. It has 8 10/100Base-TX RJ45 ports + 2G RJ45+2G SFP uplink, supports one-click HSR highly reliable seamless redundancy protocol, one-click VLAN, plug and play, and requires no configuration. This switch support HSR ring topology with one click (a ring with 3Pcs or 3Pcs+), zero recovery time, support convergence time of 50 nodes ≤6MS, 5,000 times faster than RSTP. All ports support full-line speed forwarding, its high-performance switching engine, with IP40 protection grade housing, fanless design efficient wavy aluminum alloy housing cooling surface, power input end overcurrent, over-voltage and EMC protection, RJ45 port excellent EMC protection performance, making the series of products can adapt to harsh and dangerous industrial environment. Provide reliable and convenient solutions for users' Ethernet devices networking.

SPECIFICATIONS:

| Product Name | 8-Port 100Mbps RJ45+2G RJ45+2G SFP Industrial Switch |
| --- | --- |
| Model No. | EL10822FG-SFP |
| Ethernet interface |  |
| RJ45 Port | 8*10/100BaseT(X),Automatic detection, Full/half duplex MDI/MDI-x adaptive 2*10/100/1000BaseT(X) uplink |
| Optical port | 2*1000Base-FX port SFP uplink |
| Power Parameters |  |
| Power | Connection: 1 removable 5-contact terminal block(s) Input voltage: DC12 to 52V redundant power support reverse connection protection |
| LED Indicator | Power: Green Link: Green |
| Standards |  |
| Network Protocols | IEEE802.3-10BaseT、IEEE802.3u-100BaseTX/100Base-FX、IEEE802.3x-Flow Control、IEEE802.3z-1000BaseLX、IEEE802.3ab-1000BaseTX、IEEE802.1ab、IEEE802.1D-Spanning Tree Protocol、IEEE802.1w-Rapid Spanning Tree Protoco、APS IEEE802.1Q -VLAN Tagging、 IEEE802.1p -Class of Service、IEEE802.1X-Port Based Network Access Control |
| Twisted Pair | 10BASE-T: Cat3,4,5 UTP(≤100 meter) 100BASE-TX: Cat5 or later UTP(≤100 meter) |
| One-click switcher | Switcher for HSR (RING connection) Switcher for Vlan Isolation between downlink port |
| Switch Properties |  |
| Processing Type: | Store and Forward |
| MAC Table Size | 8K |
| Packet Buffer Size | 4Mbit |
| Bandwidth | 26G |
| Switching Delay | < 10μs |
| Power Consume | ≤ 5W |
| Physical Characteristics |  |
| Protection level | IP40 |
| Enclosure | Wavy aluminum alloy |
| Installation | Push-in DIN rail |
| Weigh | 1.0kg |
| Dimensions | 150x125x53mm（length x width x height) |
| Environmental Limits |  |
| Working temperature | -40～75℃ （-40～167 °F） |
| Storage Temperature (package included) | -40 to 85°C (-40 to 185°F) |
| Ambient Relative Humidity | 5 to 95% (non-condensing) |
| Standards and Certifications |  |
| Safety | EN 62368-1(LVD) |
| EMI | FCC Part 15 Subpart B Class A，EN 55032 Class A |
| EMS | IEC(EN)61000-4-2(ESD): ±8kV Contact, ±8kV Indirect,±15kV Air IEC(EN)61000-4-3(RS):10V/m(80～1000MHz) IEC(EN)61000-4-4(EFT): power line:±4kV; data line:±2kV IEC(EN)61000-4-5(Surge): power:±6kV CM/±4kV DM; data:±4kV IEC(EN)61000-4-6(RF conduction):3V(10kHz～150kHz),10V(150kHz～80MHz) IEC(EN)61000-4-16(Common mode conduction):30V cont. 300V,1s IEC(EN )61000-4-8 |
| Shock | IEC 60068-2-27 |
| Freefall | IEC 60068-2-32 |
| Vibration | IEC 60068-2-6 |
| MTBF |  |
| Time | 300 000hrs |
| Warranty |  |
| Warranty Period | 5years |

Installation method