EL82MGI V3                               8-Port RJ45 +2G SFP L2 Managed Industrial Switch

Interface

- 8*10/100/1000BaseT(X), Automatic detection, Full/half duplex MDI/MDI-x adaptive

- 2*100/1000Base-FX Port SFP

Industrial conformance

- 12V to 52V DC, redundant power input

- -40℃ to 75℃ operating temperature

- IP40 aluminum housing

- Supports 6KV DC Ethernet ESD protection

- Supports 6KV DC EFT protection for power line

- Free Fall, Shock and Vibration Stability

- DIN-rail hardware design

EL82MGI V3 is an optical convergence integrated Managed Industrial Ethernet Switch independently developed by our company. This product is mainly developed for the application of point-to-point networks, star networks, tree networks, and chain networks in the industrial field. It supports 8-Port 10/100/1000Base-TX and 2-Port 100/1000Base-FX SFP, and the longest optical communication distance can reach 120KM. 
In addition to its compact size, it has a high-performance switching engine, a wavy aluminum alloy shell with IP40 protection level, a fanless design and efficient heat dissipation surface, and an installed power input terminal over-current, over-voltage, the EL82MGI V3 features DC12-52V redundant power inputs, DIN-rail mounting, high-level EMI/EMC capability, and an operating temperature range of -40 to 75°C available. The EL82MGI V3 has also passed a 100% burn-in test to ensure it will function reliably in the field. Therefore, it can provide reliable and convenient solutions for users’ Ethernet equipment networking.

SPECIFICATIONS:

| Product Name | 8-Port RJ45 +2G SFP L2 Managed Industrial Switch |
| --- | --- |
| Model No. | EL82MGI V3 |
| Ethernet interface |  |
| RJ45 Port | 8*10/100/1000BaseT(X),Automatic detection, Full/half duplex MDI/MDI-x adaptive |
| Optical port | 2*100/1000Base-FX SFP |
| Power Parameters |  |
| Power | Connection: 1 removable 5-contact terminal block(s) Input voltage: DC12 to 52V redundant power support reverse connection protection |
| Relay out put | 1 set of alarm output relay output |
| LED Indicator | Power indicator light: red P1, P2 Interface indicator light: Left of the RJ45: Yellow, Right: Green (Data) Optical port green Uplink/act indicator light |
| Standards |  |
| Network Protocols | IEEE802.3-10BaseT,IEEE802.3u-100BaseTX/100Base-FX,IEEE802.3x-Flow Control,IEEE802.3z-1000BaseLX,IEEE802.3ab-1000BaseTX,IEEE802.1ab Logical Link Discovery Protocol,IEEE802.1D-Spanning Tree Protocol,IEEE802.1w-Rapid Spanning Tree Protocol, APS ,IEEE802.1Q -VLAN Tagging, IEEE802.1p -Class of Service,IEEE802.1X-Port Based Network Access Control |
| Twisted Pair | 10BASE-T: Cat3,4,5 UTP(≤100 meter) 100BASE-TX: Cat5 or later UTP(≤100 meter) 1000BASE-TX: Cat6 or later UTP(≤100 meter) |
| Switch Properties |  |
| Processing Type: | Store and Forward |
| Packet forwarding rate | 17.856Mpps |
| MAC Table Size | 8K |
| Packet Buffer Size | 4.1Mbit |
| Bandwidth | 26G |
| Switching Delay | ＜10μs |
| Power Consume | ＜15W |
| Physical Characteristics |  |
| Protection level | IP40 |
| Enclosure | Wavy aluminum alloy |
| Installation | Push-in DIN rail |
| Weigh | 0.75kg |
| Dimensions | 140x116x42mm (L*W*H) |
| Environmental Limits |  |
| Working temperature | -40～75°C（-40～167°F） |
| Storage Temperature (package included) | -40 to 85°C (-40 to 185°F) |
| Ambient Relative Humidity | 5 to 95% (non-condensing) |
| Standards and Certifications |  |
| Safety | EN 62368-1(LVD) |
| EMI | FCC Part 15 Subpart B Class A EN 55032 Class A |
| EMS | IEC(EN)61000-4-2(ESD): ±8kV Contact, ±8kV Indirect,±15kV Air IEC(EN)61000-4-3(RS):10V/m(80～1000MHz) IEC(EN)61000-4-4(EFT): power line:±4kV; data line:±2kV IEC(EN)61000-4-5(Surge): power:±6kV CM/±4kV DM; data:±4kV IEC(EN)61000-4-6(RF conduction):3V(10kHz～150kHz),10V(150kHz～80MHz) IEC(EN)61000-4-16(Common mode conduction):30V cont. 300V,1s IEC(EN )61000-4-8 |
| Shock | IEC 60068-2-27 |
| Freefall | IEC 60068-2-32 |
| Vibration | IEC 60068-2-6 |
| MTBF |  |
| Time | 300 000hrs |
| Warranty |  |
| Warranty Period | 5years |

SOFTWARE:

| Port Features | IEEE 802.3x Flow control |
| --- | --- |
|  | Interface counters |
|  | Storm Control |
| Link Aggregation | Static Aggregation |
|  | Dynamic Aggregation, LACP |
|  | Supports up to 4 aggregation groups, each group up to 8 ports Load balancing |
| MAC Address Table | Static MAC Address MAC Binding |
| Mirroring | Port-based One-to-one Mirroring, Many-to-one Mirroring |
| Spanning Tree | IEEE802.1d STP |
|  | IEEE802.1w RSTP |
|  | IEEE802.1s MSTP |
| ERPS (G.8032) | Single Ring, Sub-Ring, Multi-Ring |
|  | Recovery Time < 50ms |
| LLDP | LLDP |
|  | LLDP-MED |
| Loop Protection | Supported |
| VLAN | 4094 VLAN |
|  | Port-based VLAN |
|  | GVRP |
| IGMP | Snooping IGMP V1 Snooping |
|  | IGMP V2 Snooping |
|  | IGMP V3 Snooping |
|  | Source Port Check |
|  | IGMP Fast Leave |
| Security |  |
| ACL | ACL (IP Standard ACL), IPv4/IPv6 |
|  | MAC extend ACL |
|  | IP extend ACL |
|  | L2, L3, L4 fields match |
| QoS | QoS Class, Remarking |
|  | SP, WRR queue scheduling |
|  | Ingress port-based rate-limit |
|  | Ingress priority-based rate-limit |
|  | Egress port-based rate-limit |
|  | Egress queue-based rate-limit |
|  | Policy-based QoS |
|  | Storm Control |
|  | 8 queues per port |
| 802.1X | Port-based |
| User management | Password protection |
| SNMP | SNMP V1/V2c/V3 |
| RMON | Statistics Group |
|  | History Group |
|  | Alarm Group |
|  | Events Group |
| Web management | HTTP V1.1, HTTPS |
| Firmware Upgrade | Reversion Upgrade |
| CLI | Support |
| Syslog/Debug | Support |
| Diagnosis | Ping (IPv4), ping (IPv6) |
|  | Trace route(IPv4) |
| Management |  |
| NTP | SNTP |
| Telnet | Telnet Server |
| TFTP | TFTP Client |
| SSH | SSH Server |

Dimensions

Order Information

| Specification | Model No. |
| --- | --- |
| L2 Managed Industrial Switch 8 Port 10/100/1000Base TX+2 Port 100/1000 SFP | EL82MGI V3 |