EL1160F                                     16-Port Fast Industrial Switch

Interface

- 16*10/100BaseT(X),Automatic detection, Full/half duplex MDI/MDI-x adaptive

Industrial conformance

- 12V to 52V DC, redundant power input

- -40℃ to 75℃ operating temperature

- IP40 aluminum housing

- Supports 6KV DC Ethernet ESD protection

- Supports 6KV DC EFT protection for power line

- Free Fall, Shock and Vibration Stability

- DIN-rail hardware design

EL1160F is a converging integrated industrial Ethernet un-managed switch independently developed by our company. This product is mainly developed for the application of point-to-point networks, star networks, tree networks and chain networks in the industrial field. It supports 16-Port 10/100Base-TX. This product has a high-performance switching engine, a wavy aluminum alloy shell with an IP40 protection level, a fanless design and an efficient heat dissipation surface, and over-current, over-voltage and EMC protection for the loading power input port. The RJ45 port is equipped with excellent EMC protection performance, so that the series of products can adapt to harsh and dangerous industrial environments. Therefore, it can provide reliable and convenient solutions for users’ Ethernet equipment networking.

SPECIFICATIONS:

| Model No. | EL1160F |
| --- | --- |
| Product description | 16-Port Fast unmanaged Industrial Ethernet Switch |
| Port Description | 16-Port RJ45 connector |
| Ethernet interface |  |
| RJ45 Ports | 10/100BaseT(X),Automatic detection,Full/half duplex MDI/MDI-x adaptive |
| Standards |  |
| Network Protocols | IEEE802.3 10BASE-T; 100BASE-T IEEE802.3i 10Base-T;IEEE802.3u;IEEE802.3x; |
| Twisted Pair | 10BASE-T: Cat3,4,5 UTP(≤100 meter) 100BASE-TX: Cat5 or later UTP(≤100 meter) |
| Switch Properties |  |
| Processing Type: | Store and Forward |
| MAC Table Size | 16K |
| Packet Buffer Size | 4Mbit |
| Bandwidth | 14.8G |
| Switching Delay | ＜10μs |
| Power Consume | ＜10Watts |
| Power Parameters |  |
| Power | Connection: 1 removable 5-contact terminal block(s) Input voltage: DC12 to 52V redundant dual inputs support reverse connection protection |
| LED Indicator | Power: Green Link: Green |
| Physical Characteristics |  |
| Enclosure | Wavy aluminum alloy |
| Protection level | IP40 |
| Dimensions | 150x 126x 53mm (length x width x height) |
| Weigh | 0.7kg |
| Installation | Push-in DIN rail |
| Environmental Limits |  |
| Operating Temperature | -40～75℃ （-40～185 °F） |
| Storage Temperature | -40 to 85°C (-40 to 185°F) |
| Ambient Relative Humidity | 5 to 95% (non-condensing) |
| Standards and Certifications |  |
| Safety | EN 62368-1(LVD) |
| EMI | FCC Part 15 Subpart B Class A，EN 55032 Class A |
| EMS | IEC(EN)61000-4-2(ESD): ±8kV Contact, ±8kV Indirect,±15kV Air IEC(EN)61000-4-3(RS):10V/m(80～1000MHz) IEC(EN)61000-4-4(EFT): power line:±4kV; data line:±2kV IEC(EN)61000-4-5(Surge): power:±6kV CM/±4kV DM; data:±4kV IEC(EN)61000-4-6(RF conduction):3V(10kHz～150kHz),10V(150kHz～80MHz) IEC(EN)61000-4-16(Common mode conduction):30V cont. 300V,1s IEC(EN )61000-4-8 |
| Shock | IEC 60068-2-27 |
| Freefall | IEC 60068-2-32 |
| Vibration | IEC 60068-2-6 |
| MTBF |  |
| Time | 300 000hrs |
| Warranty |  |
| Warranty Period | 5 years |

Installation method