EL448GST4MI

48-Port Gigabit + 4*10G SFP L3 Managed Industrial Switch

Product features:

- Data control: Supports 802.3x full-duplex flow control and network storm suppression

- Redundant network: supports STP, RSTP, MSTP and ERPS

- Redundancy backup: supports the virtual route redundancy protocol VRRP

- Multicast management: Support IGMP Snooping V1, V2, and V3

- VLAN: supports IEEE 802.1Q vlans to effectively isolate broadcast domains

- Link aggregation: Support static/dynamic link aggregation, provide perfect bandwidth utilization

- QOS: Support COS\DSCP, 4 queues, support WRR\SP scheduling mode

- Security management: Security management: supports ACL and 802.1x

- Management: Supports WEB, CLI, and SNMP management

- Monitoring and maintenance: Support port mirroring, interface status monitoring, log management

EL448GST4MI  switch is a high performance 48 Port gigabit RJ45 + 4 Port 10G SFP+ L3 core switch. It provides Static routing and dynamic routing with advanced hardware processing capacity. The machine adopts modular design, which supports up to 4 Port 10G SFP+, and retains the expansion ability to further support 10GE when realizing gigabit convergence or access. It supports IPv4/IPv6 hardware double stack and line speed forwarding, so that customers can easily cope with the coming IPv6 era, which provides high capacity exchange capacity. EL448GST4MI switch has the operating temperature of -40℃ ~ 75℃, has the super firmness to adapt to all kinds of harsh environment, can also be very convenient to install in the machine room cabinet. The rack-mounted characteristics and wide temperature operation make the switch a plug and play industrial-grade device, providing a reliable and convenient solution for users to network their Ethernet devices

Strong processing ability

EL448GST4MI switch is based on the powerful and stable

processing power of VCore-III MIPS-based CPU

EL448GST4MI provides up to 4.8Tbps of switching capacity

port full speed filtering and forwarding 1624Mpps

To ensure that the trunk network of large traffic data without blocking

Forwarding.

SPECIFICATIONS:

| Model No. | EL448GST4MI |
| --- | --- |
| Interface | 48 Port 10/1001000Mbps RJ45 4 Port 10G SFP+ (compatible with 1000M/2500M/10000M) 1-Port console |
| Performance Specification | Bandwidth(switch capacity): 4.8Tbps Packet Buffer Memory:32Mbit Packet Forwarding Rate: 1624Mpps MAC Address Table: 32K |
| Installation | Bracket mounted |
| Maximum Frame Size | 10240bytes |
| Flow Control | Back pressure for half duplex, IEEE 802.3x pause frame for full duplex |
| Enclosure | IP50 Aluminum housing |
| LED Indicator | POWER: Red RUN: Green Fiber: X1~X4(Green) Ethernet: Green |
| Power Input | AC110-240V |
| Power Consumption | < 60watts |
| Surge protection | ±4KV |
| Network Protocols | IEEE 802.3ad, link aggregation protocols IEEE802.3 10BASE-T; IEEE802.3i 10Base-T; IEEE802.3u;100Base-TX/FX; IEEE802.3ab 1000Base-T; IEEE802.3z 1000Base-X; IEEE 802.3ae,10G IEEE 802.3x, Full duplex Ethernet data link Laminar flow control IEEE 802.3az,EEE high efficient ethernet |
| Network cables | 10BASE-T: Cat3,4,5 UTP(≤100 meter) 100BASE-TX: Cat5 or later UTP(≤100meter) 1000BASE-TX: Cat6 or later UTP(≤100 meter) |
| Industry Standard (EMC 4) | FCC CFR47 Part 15,EN55032 IEC61000-4-2 (ESD): ±8kV (contact), ±12kV (air) IEC61000-4-3 (RS): 10V/m (80~1000MHz) IEC61000-4-4 (EFT): Power Port: ±4kV; Data Port: ±2kV IEC61000-4-5 (Surge): Power Port: ±2kV/DM, ±4kV/CM; Data Port: ±2kV IEC61000-4-6 (CS): 3V (10kHz-150kHz); 10V (150kHz-80MHz) IEC61000-4-16 (Common mode conduction): 30V (cont.), 300V (1s) |
| MTBF | 300,000hours |
| Dimensions (W x D x H) | 442X285X45mm |
| Weight | Product Weight: 5.0KGS Packing Weight: 6.0KGS |
| Working Environment | Working temperature: -40～75℃ ; Storage temperature: -40～75℃ Relative Humidity: 5%~95 %( no condensation) |

SOFTWARE:

| Interface | IEEE802.3X (Full-duplex), Port temperature protection setting Port green Ethernet Energy-saving setting Broadcast storm control based on port speed. The speed limit of the message flow in the access port. The minimum particle size is 64Kbps. |
| --- | --- |
| Layer 3 Features | L3 network management, IPV4/IPV6 management L3 soft routing forwarding, Static route, Default route @ 128 pcs, APR@1024pcs |
| VLAN | 4K VLAN based on port, IEEE802.1q VLAN based on the protocol VLAN based on MAC VLAN based on IP Voice VLAN, QinQ configuration Port configuration of Access, Trunk, Hybrid |
| Port Aggregation | LACP, Static aggregation Max 14 aggregation groups and 16 ports per group. |
| Spanning Tree | STP (IEEE802.1d), RSTP (IEEE802.1w), MSTP (IEEE802.1s) |
| Industrial Ring Network Protocol | G.8032 (ERPS), Recovery time less than 20ms 250 Ring at most, Max 250 devices per ring. Multicast |
| Multicast | MLD Snooping v1/v2,Multicast VLAN Registration (MVR) IGMP Snooping v1/v2, Max 1024 multicast groups, Fast log out |
| Port Mirroring | Bidirectional data mirroring based on port |
| QoS | SP（Strict Priorty） WFQ (Weighted Fair Queuing) WRR (Weighted Round Robin) Random Early Detect (RED) WeightedRandomEarlyDetection (WRED) Head Of Line Best-Effort Service First come first served 802.1p Cos/Tos，QOS Differentiated Service |
| ACL | Port-based Issuing ACL,ACL based on port and VLAN L2 to L4 packet filtering, matching first 80bytes message. Provide ACL based on MAC, Destination MAC address, IP Source, Destination IP, IP Protocol Type, TCP/UDP Port, TCP/UDP Port Range, and VLAN, etc. |
| Security | Support IP ACL，MAC ACL，Vlan ACL,support ACL based on L3 L4 IP+MAC Port binding Support IP Source Guard, RP Detection Support IEEE 802.1x port verification, support Radius |
| DHCP | DHCP Client, DHCP Snooping, DHCP Server, DHCP Relay |
| IP routing | Supports port-based input/output bandwidth management. Supports static routes. IPV6 Supports the virtual route redundancy protocol VRRP. Dynamic routes such as RIP, RIPng, OSPF, and OSPFv3 are supported. |
| System maintenance | Supports configuration file uploading and downloading. Upload the upgrade package. Supports factory restore on WEB, Supports the system log function. Support Ping 、tracert、cable detection and other diagnostic functions. Supports WEB management (HTTP and HTTPS). Supports CLI management. Supports Telnet remote management. Supports SSH 2.0. Supports SNMP network management. Support RMON; Supports NTP clients. |

Ordering Information

| EL448GST4MI | 48 Port Gigabit RJ45+ 4*10G SFP L3 Managed Industrial switch |
| --- | --- |