EL242GSPMI

24-Port RJ45+2G SFP L2 Managed Industrial Switch

Interface

- 24-port 10/100/1000Base-Tx RJ-45 with auto negotiation and auto-MDI/ MDI-X function

- 2G SFP uplink (automatically 100Mbps/1000Mbps SFP module)

- 1-24 Port PoE output,each max.30W

- PoE budget: 380W max.

Industrial conformance

- AC110-240V redundant power input

- - 40℃ to 75℃ operating temperature

- IP40 aluminum housing

- Supports ESD protection: contact 8KV, air 12KV

- Supports Surge protection: CM±4KV, DM±2KV

- Free Fall, Shock and Vibration Stability

- 19“ Rak-mountable hardware design

EL242GSPMI an hardened 26-Port Industrial Ethernet Switch, provides non-blocking wire-speed performance in harsh industrial environment. It also provides 24-Port 10/100/1000BASE-TX RJ45(PoE) +2G SFP delivered in an IP40 rugged, strong case with redundant power system. It’s suitable for such applications as surveillance system deployments, control management and wireless service in climatically demanding environments with wide temperature range from -40℃ to 75℃

The EL242GSPMI provides the flexibility to all kinds of 10/100/1000Mbps Ethernet Media on SFP port and performs highly stable fiber performance

The EL242GSPMI is packaged in a compact IP40 case that allows bracket mounting to cabinet. It provides power supply AC100-240V input

SPECIFICATIONS:

| Model No. | EL242GSPMI |
| --- | --- |
| 1000Base-TX Port | 24 Port 10/100/1000 RJ45 |
| 1000Base-FX Port | 2G SFP 100/1000 |
| PoE Specification | PoE ports:1-24 port support PoE Max. 15.4 watts (IEEE 802.3af) Max. 30 watts (IEEE 802.3at) Power Pin Assignment:1/2+;3/6- PoE budget: Max.380W |
| Performance Specification | Bandwidth: 56Gbps Packet Buffer Memory: 12Mbit Packet Forwarding Rate: 42Mpps MAC Address Table: 8K |
| Installation | Rak mounted |
| Maximum Frame Size | 10240bytes packet size |
| Flow Control | Back pressure for half duplex, IEEE 802.3x pause frame for full duplex |
| Enclosure | IP40 Aluminum housing |
| LED Indicator | Power: Red Fiber: Link (Green) Ethernet: Yellow |
| Power Input | AC110-240V Supports built-in over-current 4.0A protection Supports reverse connection protection |
| Power Consumption | < 400W |
| Surge protection | ± 4KV |
| Network Protocols | IEEE802.3 10BASE-T; IEEE802.3i 10Base-T; IEEE802.3u;100Base-TX/FX; IEEE802.3ab 1000Base-T; IEEE802.3z 1000Base-X; IEEE802.3x; |
| Network cables | 10BASE-T: Cat3,4,5 UTP(≤100 meter) 100BASE-TX: Cat5 or later UTP(≤100meter) 1000BASE-TX: Cat6 or later UTP(≤100 meter) |
| Industry Standard | FCC CFR47 Part 15,EN55032 Class A IEC61000-4-2 (ESD): ±8kV (contact), ±12kV (air) IEC61000-4-3 (RS): 10V/m (80~1000MHz) IEC61000-4-4 (EFT): Power Port: ±4kV; Data Port: ±2kV IEC61000-4-5 (Surge): Power Port: ±2kV/DM, ±4kV/CM; Data Port: ±2kV IEC61000-4-6 (CS): 3V (10kHz-150kHz); 10V (150kHz-80MHz) IEC61000-4-16 (Common mode conduction): 30V (cont.), 300V (1s) |
| MTBF | 300,000hours |
| Dimensions (W x D x H) | 442×325×44mm |
| Weight | Product Weight: 4.5KG Packing Weight: 5.0KG |
| Working Environment | Working temperature: -40～75℃ ; Storage temperature: -40～75℃ Relative Humidity: 5%~95 %( no condensation) |

SOFTWARE:

| Port Features | IEEE 802.3x Flow control |
| --- | --- |
|  | Interface counters |
|  | Storm Control |
| Link Aggregation | Static Aggregation |
|  | Dynamic Aggregation, LACP |
|  | Supports up to 4 aggregation groups, each group up to 8 ports Load balancing |
| MAC Address Table | Static MAC Address MAC Binding |
| Mirroring | Port-based One-to-one Mirroring, Many-to-one Mirroring |
| Spanning Tree | IEEE802.1d STP |
|  | IEEE802.1w RSTP |
|  | IEEE802.1s MSTP |
| ERPS (G.8032) | Single Ring, Sub-Ring, Multi-Ring |
|  | Recovery Time < 50ms |
| LLDP | LLDP |
|  | LLDP-MED |
| Loop Protection | Supported |
| VLAN | 4094 VLAN |
|  | Port-based VLAN |
|  | GVRP |
| Protocol VLAN | MAC-based VLAN |
|  | IP Subnet-based VLAN |
|  | Voice VLAN |
| Port Isolate | Support |
| IGMP | Snooping IGMP V1 Snooping |
|  | IGMP V2 Snooping |
|  | IGMP V3 Snooping |
|  | Source Port Check |
|  | IGMP Fast Leave |
|  | IGMP Filter |
| MLD Snooping | MLD V1 Snooping |
|  | MLD V2 Snooping |
|  | Source Port Check |
|  | MLD Fast Leave |
|  | MLD Filter |
| Security |  |
| ACL | ACL (IP Standard ACL), IPv4/IPv6 |
|  | MAC extend ACL |
|  | IP extend ACL |
|  | L2, L3, L4 fields match |
|  | log, redirection, mirroring, speed limit |
| QoS | QoS Class, Remarking |
|  | SP, WRR queue scheduling |
|  | Ingress port-based rate-limit |
|  | Ingress priority-based rate-limit |
|  | Egress port-based rate-limit |
|  | Egress queue-based rate-limit |
|  | Policy-based QoS |
|  | Storm Control |
|  | 8 queues per port |
| 802.1X | Port-based |
| User management | Password protection |
| SNMP | SNMP V1/V2c/V3 |
| RMON | Statistics Group |
|  | History Group |
|  | Alarm Group |
|  | Events Group |
| Web management | HTTP V1.1, HTTPS |
| Firmware Upgrade | Reversion Upgrade |
| IPv4/IPv6 | Support |
| CLI | Support |
| Syslog/Debug | Support |
| Diagnosis | Ping (IPv4), ping (IPv6) |
|  | Trace route(IPv4) |
| Management |  |
| NTP | NTP Client |
| DHCP | DHCPv4 Client, DHCPv6 Client |
| DHCP Snooping | DHCP snooping table |
|  | Trust Port |
| Telnet | Telnet Server |
| TFTP | TFTP Client |
| SSH | SSH Server |

Dimensions

Ordering Information

| EL242GSPMI | 24-Port 10/100/100BASE-TX+2G SFP Managed Industrial PoE Switch |
| --- | --- |