EL284CMGPI                               24-Port RJ45 (PoE)+4G Combo L2 Managed Industrial PoE Switch

Interface

- 24*10/100/1000BaseT(X), Automatic detection, Full/half duplex MDI/MDI-x adaptive

- 4*100/1000 SFP/Copper (combo)

- 16 Port 802.3af/at PoE

- 8 Port 802.3/bt PoE

- PoE output max.400W

Industrial conformance

- 48V to 60V DC, redundant power input

- -40℃ to 75℃ operating temperature

- IP40 aluminum housing

- Supports 6KV DC Ethernet ESD protection

- Supports 6KV DC EFT protection for power line

- Free Fall, Shock and Vibration Stability

- Rack-mountable design

EL284CMGPI is an optical convergence integrated Managed Industrial Ethernet Switch independently developed by our company. This product is mainly developed for the application of point-to-point networks, star networks, tree networks, and chain networks in the industrial field. It supports 24-Port 10/100/1000Base-TX and 4G Combo (SFP/Copper), and the longest optical communication distance can reach 120KM. 
In addition to its compact size, it has a high-performance switching engine, a wavy aluminum alloy shell with IP40 protection level, a fanless design and efficient heat dissipation surface, and an installed power input terminal over-current, over-voltage, the EL284CMGPI features DC48-60V redundant power inputs, 19” rack mounting, high-level EMI/EMC capability, and an operating temperature range of -40 to 75°C available. The EL284CMGPI has also passed a 100% burn-in test to ensure it will function reliably in the field. Therefore, it can provide reliable and convenient solutions for users’ Ethernet equipment networking

SPECIFICATIONS:

| Product Name | 24-Port RJ45 (PoE)+4G Combo L2 Managed Industrial PoE Switch |
| --- | --- |
| Model No. | EL284CMGPI |
| Ethernet interface |  |
| RJ45 Port | 24*10/100/1000BaseT(X),Automatic detection, Full/half duplex MDI/MDI-x adaptive |
| Optical port | 4*100/1000 SFP/RJ45 Combo（Support DDM） |
| PoE | PoE Port: 24 (Port 1-16 af/at PoE, Port 17-24 bt PoE) PoE budget: 400W max IEEE 802.3af maximum power 15.4W IEEE 802.3at maximum power 30W IEEE 802.3bt maximum power 60W PoE port automatically identifies af/at/bt PoE power supply pin: corresponding pin V+ (1, 2) V- (3, 6) |
| Power Parameters |  |
| Power | Input voltage: DC48 to 60V redundant power support PoE enabled, input voltage ≥48VDC required DC input with reverse-polarity & over-current protection |
| Relay output | 1 set of alarm output relay output |
| LED Indicator | Power: Green PWR LED System: Green SYS LED Port: Green Link/Act for RJ45; Green Ulink/Act for SFP combo ports Alarm: Yellow alarm LED |
| Standards |  |
| Network Protocols | IEEE 802.3 for 10BaseT IEEE 802.3u for 100BaseT(X) and 100BaseFX IEEE 802.3ab for 1000BaseT(X) IEEE 802.3z for 1000BaseSX/LX/LHX/ZX IEEE 802.3x for flow control IEEE 802.1D-2004 for Spanning Tree Protocol IEEE 802.1w for Rapid Spanning Tree Protocol IEEE 802.1s for Multiple Spanning Tree Protocol IEEE 802.1p for Class of Service IEEE 802.1Q for VLAN Tagging IEEE 802.1X for authentication IEEE 802.3ad for Port Trunk with LACP |
| Twisted Pair | 10BASE-T: Cat3,4,5 UTP(≤100 meter) 100BASE-TX: Cat5 or later UTP(≤100 meter) 1000BASE-TX: Cat6 or later UTP(≤100 meter) |
| Switch Properties |  |
| Processing Type: | Store and Forward |
| Packet forwarding rate | 41.664Mpps |
| MAC Table Size | 8K |
| Packet Buffer Size | 4.1Mbits |
| Bandwidth | 56G |
| Switching Delay | ＜10μs |
| Power Consume | ＜425W (full load) |
| Physical Characteristics |  |
| Protection level | IP40 |
| Enclosure | Wavy aluminum alloy |
| Installation | Push-in DIN rail |
| Weigh | 6.0kg |
| Dimensions | 440x300x44mm (L*W*H) |
| Environmental Limits |  |
| Working temperature | -40～75°C（-40～167°F） |
| Storage Temperature (package included) | -40 to 85°C (-40 to 185°F) |
| Ambient Relative Humidity | 5 to 95% (non-condensing) |
| Standards and Certifications |  |
| Safety | EN 62368-1(LVD) |
| EMI | FCC Part 15 Subpart B Class A EN 55032 Class A |
| EMS | IEC(EN)61000-4-2(ESD): ±8kV Contact, ±8kV Indirect,±15kV Air IEC(EN)61000-4-3(RS):10V/m(80～1000MHz) IEC(EN)61000-4-4(EFT): power line:±4kV; data line:±2kV IEC(EN)61000-4-5(Surge): power:±6kV CM/±4kV DM; data:±4kV IEC(EN)61000-4-6(RF conduction):3V(10kHz～150kHz),10V(150kHz～80MHz) IEC(EN)61000-4-16(Common mode conduction):30V cont. 300V,1s IEC(EN )61000-4-8 |
| Shock | IEC 60068-2-27 |
| Freefall | IEC 60068-2-32 |
| Vibration | IEC 60068-2-6 |
| MTBF |  |
| Time | 300 000hrs |
| Warranty |  |
| Warranty Period | 5years |

SOFTWARE:

| Port Features | IEEE 802.3x Flow control |
| --- | --- |
|  | Interface counters |
|  | Storm Control |
| Link Aggregation | Static Aggregation |
|  | Dynamic Aggregation, LACP |
|  | Supports up to 4 aggregation groups, each group up to 8 ports Load balancing |
| MAC Address Table | Static MAC Address MAC Binding |
| Mirroring | Port-based One-to-one Mirroring, Many-to-one Mirroring |
| Spanning Tree | IEEE802.1d STP |
|  | IEEE802.1w RSTP |
|  | IEEE802.1s MSTP |
| ERPS (G.8032) | Single Ring, Sub-Ring, Multi-Ring |
|  | Recovery Time < 50ms |
| LLDP | LLDP |
|  | LLDP-MED |
| Loop Protection | Supported |
| VLAN | 4094 VLAN |
|  | Port-based VLAN |
|  | GVRP |
| IGMP | Snooping IGMP V1 Snooping |
|  | IGMP V2 Snooping |
|  | IGMP V3 Snooping |
|  | Source Port Check |
|  | IGMP Fast Leave |
| Security |  |
| ACL | ACL (IP Standard ACL), IPv4/IPv6 |
|  | MAC extend ACL |
|  | IP extend ACL |
|  | L2, L3, L4 fields match |
| QoS | QoS Class, Remarking |
|  | SP, WRR queue scheduling |
|  | Ingress port-based rate-limit |
|  | Ingress priority-based rate-limit |
|  | Egress port-based rate-limit |
|  | Egress queue-based rate-limit |
|  | Policy-based QoS |
|  | Storm Control |
|  | 8 queues per port |
| 802.1X | Port-based |
| User management | Password protection |
| SNMP | SNMP V1/V2c/V3 |
| RMON | Statistics Group |
|  | History Group |
|  | Alarm Group |
|  | Events Group |
| Web management | HTTP V1.1, HTTPS |
| Firmware Upgrade | Reversion Upgrade |
| CLI | Support |
| Syslog/Debug | Support |
| Diagnosis | Ping (IPv4), ping (IPv6) |
|  | Trace route(IPv4) |
| Management |  |
| NTP | SNTP |
| Telnet | Telnet Server |
| TFTP | TFTP Client |
| SSH | SSH Server |

Dimensions

Order Information

| Specification | Model No. |
| --- | --- |
| L2 Managed Industrial PoE+ Switch 24 Port 10/100/1000Base TX(PoE)+4G Combo | EL284CMGPI |