EL1042F-SFP                                4-Port RJ45+2 SFP Fast Industrial Switch

Interface

- 4*10/100BaseT(X),Automatic detection, Full/half duplex MDI/MDI-x adaptive

- 2*100Base-FX Port , SFP

Industrial conformance

- 9V to 52V DC, redundant power input

- -40℃ to 75℃ operating temperature

- IP40 aluminum housing

- Supports 6KV DC Ethernet ESD protection

- Supports 6KV DC EFT protection for power line

- Free Fall, Shock and Vibration Stability

- DIN-rail hardware design

EL1042F-SFP is an integrated industrial switch (Industrial media converter) with 4 Port RJ45 100Mbps+2 Port SFP uplink. This product is mainly developed for the application of point-to-point networks, star networks, tree networks and chain networks in the industrial field. The longest optical communication distance can reach 120KM. Its high-performance switching engine has an IP40 protection-grade casing, a fanless design and an efficient corrugated aluminum alloy casing heat dissipation surface. The power input end has over-current, over-voltage and EMC protection, and the RJ45 port has excellent EMC protection performance, making this series of products able to adapt to harsh and dangerous industrial environments. Provide reliable and convenient solutions for users' Ethernet device networking.

SPECIFICATIONS:

| Product Name | 4 Port RJ45+2 Port SFP Industrial Switch |
| --- | --- |
| Model No. | EL1042F-SFP |
| Ethernet interface |  |
| RJ45 Port | 4*10/100BaseT(X),Automatic detection, Full/half duplex MDI/MDI-x adaptive |
| Optical port | 2*100Base-FX SFP |
| Power Parameters |  |
| Power | Connection: 1x removable 5-contact terminal block(s) Input voltage: DC9 to 52V redundant power support reverse connection protection |
| LED Indicator | Power: Green Link: Green |
| Standards |  |
| Network Protocols | IEEE802.3 10BASE-T;100BASE-T IEEE802.3i 10Base-T;IEEE802.3u; IEEE802.3x; |
| Twisted Pair | 10BASE-T: Cat3,4,5 UTP(≤100 meter) 100BASE-TX: Cat5 or later UTP(≤100 meter) |
| Switch Properties |  |
| Processing Type: | Store and Forward |
| MAC Table Size | 2K |
| Packet Buffer Size | 2Mbit |
| Bandwidth | 1.6G |
| Switching Delay | ＜10μs |
| Power Consume | ＜5W |
| Physical Characteristics |  |
| Protection level | IP40 |
| Enclosure | Wavy aluminum alloy |
| Installation | Push-in DIN rail |
| Weigh | 0.4kg |
| Dimensions | 115x70x30mm（length x width x height) |
| Environmental Limits |  |
| Working temperature | -40～75℃ （-40～167 °F） |
| Storage Temperature (package included) | -40 to 85°C (-40 to 185°F) |
| Ambient Relative Humidity | 5 to 95% (non-condensing) |
| Standards and Certifications |  |
| Safety | EN 62368-1(LVD) |
| EMI | FCC Part 15 Subpart B Class A, EN 55032 Class A |
| EMS | IEC(EN)61000-4-2(ESD): ±8kV Contact, ±8kV Indirect,±15kV Air IEC(EN)61000-4-3(RS):10V/m(80～1000MHz) IEC(EN)61000-4-4(EFT): power line:±4kV; data line:±2kV IEC(EN)61000-4-5(Surge): power:±6kV CM/±4kV DM; data:±4kV IEC(EN)61000-4-6(RF conduction):3V(10kHz～150kHz),10V(150kHz～80MHz) IEC(EN)61000-4-16(Common mode conduction):30V cont. 300V,1s IEC(EN )61000-4-8 |
| Shock | IEC 60068-2-27 |
| Freefall | IEC 60068-2-32 |
| Vibration | IEC 60068-2-6 |
| MTBF |  |
| Time | 300 000hrs |
| Warranty |  |
| Warranty Period | 5years |

Installation method

Booking Information:

| Model No. | Specification |
| --- | --- |
| EL1042F-SFP | 4 Port 10/100 Base -T +2-Port optical SFP Fast Industrial Switch |
| EL1042F-SC2 | 4 Port 10/100 Base -T +2-Port optical 1*9 (Multi Mode, (Tx/Rx1310nm), Dual fiber, SC2) Fast Industrial Switch, ST/FC Interface is optional |
| EL1042F-SC20 | 4 Port 10/100 Base -T +2-Port optical 1*9 (Single Mode, (Tx/Rx1310nm), Dual fiber, SC20) Fast Industrial Switch, ST/FC Interface is optional |
| EL1042F-SC20A | 4 Port 10/100 Base -T +2-Port optical 1*9 (Single Mode, (Rx1310nm/A), Single fiber, SC20) Fast Industrial Switch, ST/FC Interface is optional |
| EL1042F-SC20B | 4 Port 10/100 Base -T +2-Port optical 1*9 (Single Mode, (Tx1550nm/B), Single fiber, SC20) Fast Industrial Switch, ST/FC Interface is optional |